Weiver/WeiverPlayer API Manual

[Introduction]

.

There are three main ways to control Weiver/WeiverPlayer.

1. In the case of WeiverPlayer, control using a keypad.

2. Control using WeiverEx program.

3. Control using API.

The 1st and 2nd methods are general methods.

Method 3 is a method of controlling the command using a UDP packet.

Way to control using API supports Weiver 1.0, Weiver Player 1.0, Weiver 2.0, and Weiver Player 2.0.

It only supports playback functions and does not support recording functions.

HD Radio™ is only supported by Weiver 2.0 and Weiver Player 2.0.

[Weiver/WeiverPlayer API Operation Mechanism]

WeiverEx software uses UDP packets to control by IP connecting with Weiver/WeiverPlayer software.

API discloses API example programs and packet information because its purpose is to operate Weiver/WeiverPlayer without WeiverEx program.

You can download the reference sources from the LUMANTEK website.

Following is API Packet Structure of Weiver/WeiverPlayer. (Reference : WvPlayerRemoteAPI.h)

#define	WEIVER_PLAYER_ID			0x057b50b7

#define	WEIVER_PLAYER_API_RX_PORT		50504

#define	REMOTE_STRING_SUCCESS _T("Received")

#define	REMOTE_STRING_INVALIDE_PARAM _T("Invalid Param")

typedef enum _WvPlayerRemoteCMD_e

{

WV_PLAYER_REMOTE_CMD_SET_STRING,

WV_PLAYER_REMOTE_CMD_MAX

}WvPlayerRemoteCMD_t;

typedef struct _WvPlayerRemotePacket_t

{

unsigned int u4_ID;// WEIVER_PLAYER_ID

unsigned int u4_Reserved;//WV_PLAYER_REMOTE_CMD_SET_STRING

unsigned int u4_DataSize;

unsigned char u1a_Data[1000];

}WvPlayerRemotePacket_t, *WvPlayerRemotePacket_tp;

UDP Port: 50504 is left open to support API on Weiver/WeiverPlayer.

So, when you receive data from the WvPlayerRemotePacket structure, Weiver/WeiverPlayer executes the command.

The released source is an example of a Window program (applicable to other OSs)

Before explaining the command list, the example program (WeiverPlayerRemoteAPI.exe) makes five assumptions.

Assumption 1. Weiver/WeiverPlayer IP is 192.168.100.1

Assumption 2. The port number opened by Weiver/WeiverPlayer for API support is 50504. (Fixed)

Assumption 3. The port number left open for UDP reception is 50000.(Changes)

Assumption 4. Each letter byte has a size of 2 bytes.

Assumption 5. Time Out occurs if a response is not received after sending UDP.
[Data Packet Structure]

| connect 50000 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 26 | L”connect 50000” |
| Size(Byte) | 4 | 4 | 4 | 13x2 |

| disconnect |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 20 | L“disconnect” |
| Size(Byte) | 4 | 4 | 4 | 10x2 |

| rescandisk |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 20 | L“rescandisk” |
| Size(Byte) | 4 | 4 | 4 | 10x2 |
| openfile test.wpj |  |  |  |  |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 34 | L“openfile test.wpj” |
| Size(Byte) | 4 | 4 | 4 | 17x2 |

| get openfile |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 24 | L“get openfile” |
| Size(Byte) | 4 | 4 | 4 | 12x2 |

| play start |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 20 | L“play start” |
| Size(Byte) | 4 | 4 | 4 | 10x2 |

| play stop |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 18 | L“play stop” |
| Size(Byte) | 4 | 4 | 4 | 9x2 |

| play pause |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 20 | L“play pause” |
| Size(Byte) | 4 | 4 | 4 | 10x2 |

| play resume |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 22 | L” play resume” |
| Size(Byte) | 4 | 4 | 4 | 11x2 |

| get play_status |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 30 | L“get play_status” |
| Size(Byte) | 4 | 4 | 4 | 15x2 |
| set freq 400000000 |  |  |  |  |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 36 | L“set freq 400000000” |
| Size(Byte) | 4 | 4 | 4 | 18x2 |
| set freq 400000000 hz |  |  |  |  |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 42 | L“set freq 400000000 hz” |
| Size(Byte) | 4 | 4 | 4 | 21x2 |
| set freq 400000 khz |  |  |  |  |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 38 | L“set freq 400000 khz” |
| Size(Byte) | 4 | 4 | 4 | 19x2 |
| set freq 400 mhz |  |  |  |  |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 32 | L“set freq 400 mhz” |
| Size(Byte) | 4 | 4 | 4 | 16x2 |

| get freq |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 16 | L“get freq” |
| Size(Byte) | 4 | 4 | 4 | 8x2 |

| get gain_min |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 24 | L“get gain_min” |
| Size(Byte) | 4 | 4 | 4 | 12x2 |

| get gain_max |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 24 | L“get gain_max” |
| Size(Byte) | 4 | 4 | 4 | 12x2 |

| get gain |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 16 | L“get gain” |
| Size(Byte) | 4 | 4 | 4 | 8x2 |

| get rf_out_level 1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 36 | L“get rf_out_level 1” |
| Size(Byte) | 4 | 4 | 4 | 18x2 |

| set gain 100 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 24 | L“set gain 100” |
| Size(Byte) | 4 | 4 | 4 | 12x2 |

| set gain -100 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 26 | L“set gain -100” |
| Size(Byte) | 4 | 4 | 4 | 13x2 |

| get play_time_total |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 38 | L“get play_time_total” |
| Size(Byte) | 4 | 4 | 4 | 19x2 |

| get play_time_current |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 42 | L“get play_time_current” |
| Size(Byte) | 4 | 4 | 4 | 21x2 |

| set play_time_current 5 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 46 | L“set play_time_current 5” |
| Size(Byte) | 4 | 4 | 4 | 23x2 |

| get play_section |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 32 | L“get play_section” |
| Size(Byte) | 4 | 4 | 4 | 16x2 |

| set play_section 10 20 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 44 | L“set play_section 10 20” |
| Size(Byte) | 4 | 4 | 4 | 22x2 |

| get spec_inv |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 24 | L“get spec_inv” |
| Size(Byte) | 4 | 4 | 4 | 12x2 |

| set spec_inv off |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 32 | L“set spec_inv off” |
| Size(Byte) | 4 | 4 | 4 | 16x2 |

| set spec_inv on |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 30 | L“set spec_inv on” |
| Size(Byte) | 4 | 4 | 4 | 15x2 |

| get sample_rate |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 30 | L“get sample_rate” |
| Size(Byte) | 4 | 4 | 4 | 15x2 |

| set sample_rate 10000000 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 48 | L“set sample_rate 10000000” |
| Size(Byte) | 4 | 4 | 4 | 24x2 |

| set sample_rate 10000000 hz |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 54 | L“set sample_rate 10000000 hz” |
| Size(Byte) | 4 | 4 | 4 | 27x2 |

| set sample_rate 10 mhz |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 44 | L“set sample_rate 10 mhz” |
| Size(Byte) | 4 | 4 | 4 | 22x2 |

| set sample_rate 10000 khz |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 54 | L“set sample_rate 10000 khz” |
| Size(Byte) | 4 | 4 | 4 | 27x2 |

| set trigger_out_level low |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 50 | L“set trigger_out_level low” |
| Size(Byte) | 4 | 4 | 4 | 25x2 |

| set trigger_out_level high |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 52 | L“set trigger_out_level low” |
| Size(Byte) | 4 | 4 | 4 | 26x2 |

| get trigger_out_level |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 42 | L“get trigger_out_level” |
| Size(Byte) | 4 | 4 | 4 | 21x2 |

| set extclk off |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 28 | L“set extclk off” |
| Size(Byte) | 4 | 4 | 4 | 14x2 |

| set extclk on |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 26 | L“set extclk on” |
| Size(Byte) | 4 | 4 | 4 | 13x2 |

| get extclk |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 20 | L“get extclk” |
| Size(Byte) | 4 | 4 | 4 | 10x2 |

| get trigger_in |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 28 | L“get trigger_in” |
| Size(Byte) | 4 | 4 | 4 | 14x2 |

| set trigger_in off |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 36 | L“set trigger_in off” |
| Size(Byte) | 4 | 4 | 4 | 18x2 |

| set trigger_in on |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 34 | L“set trigger_in on” |
| Size(Byte) | 4 | 4 | 4 | 17x2 |

| set current_directory d:\ |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 50 | L“set current_directory d:\” |
| Size(Byte) | 4 | 4 | 4 | 25x2 |

| set current_directory d:\WeiverData |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 70 | L“set current_directory d:\WeiverData” |
| Size(Byte) | 4 | 4 | 4 | 35x2 |

| get current_directory |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 42 | L“get current_directory” |
| Size(Byte) | 4 | 4 | 4 | 21x2 |

| get file_count |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 28 | L“get file_count” |
| Size(Byte) | 4 | 4 | 4 | 14x2 |

| get filename 0 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 28 | L“get filename 0” |
| Size(Byte) | 4 | 4 | 4 | 14x2 |

| get folder_count |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 32 | L“get folder_count” |
| Size(Byte) | 4 | 4 | 4 | 16x2 |

| get foldername 0 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 32 | L“get foldername 0” |
| Size(Byte) | 4 | 4 | 4 | 16x2 |

| set power -200 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 28 | L“set power -200” |
| Size(Byte) | 4 | 4 | 4 | 14x2 |

| get power |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 18 | L“get power” |
| Size(Byte) | 4 | 4 | 4 | 9x2 |

| get power_min |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 26 | L“get power_min” |
| Size(Byte) | 4 | 4 | 4 | 13x2 |

| get power_max |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 26 | L“get power_max” |
| Size(Byte) | 4 | 4 | 4 | 13x2 |

| get freq_min |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 24 | L“get freq_min” |
| Size(Byte) | 4 | 4 | 4 | 12x2 |

| get freq_max |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | ID | Reserved | Data Size | Data |
| Data | 0x057b50b7 | 0 | 24 | L“get freq_max” |
| Size(Byte) | 4 | 4 | 4 | 12x2 |

Command Description

| Command | connect <parameter> |
| --- | --- |
| Support | Weiver 1.0 : O Weiver Player 1.0 : O Weiver 2.0 : O Weiver Player 2.0 : O HD Radio™ : O |
| Description | It is a command to establish a connection with the equipment. <parameter> arbitrary port number. For example, the connect 50000 command sends UDP packets to the 50000 port number when the Weiver equipment sends a response. If the equipment is under control with WeiverEx or HD Radio™ S/W, the connect command fails. If the connect command fails, send the disconnect command and send the connect command. You must specify any incoming port number that is open. |

| Command | disconnect |
| --- | --- |
| Support | Weiver 1.0 : O Weiver Player 1.0 : O Weiver 2.0 : O Weiver Player 2.0 : O HD Radio™ : O |
| Description | Command to disconnect from the equipment If RF transmission is in progress, RF transmission is automatically stopped. |

| Command | rescandisk |
| --- | --- |
| Support | Weiver 1.0 : O Weiver Player 1.0 : O Weiver 2.0 : O Weiver Player 2.0 : O HD Radio™ : O |
| Description | Command to renew a disk drive connected to the equipment |

| Command | openfile <parameter> |
| --- | --- |
| Support | Weiver 1.0 : O Weiver Player 1.0 : O Weiver 2.0 : O Weiver Player 2.0 : O HD Radio™ : O |
| Description | command to set up a file to be played on the equipment. <parameter> Arbitrary file name Before using the openfile command, go to the folder where the file is located. Refer to set current_directory for folder movement related commands. The openfile command fails when playing. To stop playing, refer to the play command. In the case of the Weiver file, it is automatically set to the central frequency and power at the time of recording. In the case of the Weiver file, refer to the set freq command and set gain command for adjusting central frequency and power. HD Radio™ FM files are automatically set to 87.9 MHz, and output power is set to the origin power of the files. HD Radio™ AM files are automatically set to 830KHz, and output power is set to the origin power of the files. For adjusting central frequency and power of HD Radio™ files, refer to the set freq command and set power command. |

| Command | get openfile |
| --- | --- |
| Support | Weiver 1.0 : O Weiver Player 1.0 : O Weiver 2.0 : O Weiver Player 2.0 : O HD Radio™ : O |
| Description | Returns the open file name. |

| Command | play start play stop play pause play resume |
| --- | --- |
| Support | Weiver 1.0 : O Weiver Player 1.0 : O Weiver 2.0 : O Weiver Player 2.0 : O HD Radio™ : O |
| Description | play start : Start RF transmission. play stop : Stop RF transmission. play pause: Pause RF transmission. play resume: Start the paused RF transmission. Use play-related commands after openfile. |

| Command | get play_status |
| --- | --- |
| Support | Weiver 1.0 : O Weiver Player 1.0 : O Weiver 2.0 : O Weiver Player 2.0 : O HD Radio™ : O |
| Description | Return the play status of the equipment. |

| Command | set freq <parameter> set freq <parameter> hz set freq <parameter> khz set freq <parameter> mhz |
| --- | --- |
| Support | Weiver 1.0 : O Weiver Player 1.0 : O Weiver 2.0 : O Weiver Player 2.0 : O HD Radio™ : O |
| Description | Set the frequency of the equipment. <parameter> Any frequency. The openfile command does not need to be changed specifically because the sample frequency and center frequency of the file are automatically set. And for HD Radio™, FM files are automatically set to 87.9 MHz and AM files to 830 KHz. Use the set freq command to set the frequency. |

| Command | get freq |
| --- | --- |
| Support | Weiver 1.0 : O Weiver Player 1.0 : O Weiver 2.0 : O Weiver Player 2.0 : O HD Radio™ : O |
| Description | Return the frequency of the equipment. |

| Command | set gain <parameter> |
| --- | --- |
| Support | Weiver 1.0 : O Weiver Player 1.0 : O Weiver 2.0 : O Weiver Player 2.0 : O HD Radio™ : X |
| Description | Command to adjust the RF transmission power of the equipment. <parameter> Arbitrary gain value. When applying gain +10.0 dB, <parameter> is 100. When applying gain -12.3 dB, <parameter> is -123. HD Radio™ files are not supported. Refer to the set power command for HD Radio™ files. |

| Command | get gain get gain_min get gain_max |
| --- | --- |
| Support | Weiver 1.0 : O Weiver Player 1.0 : O Weiver 2.0 : O Weiver Player 2.0 : O HD Radio™ : X |
| Description | get gain : Return RF transmission gain. get gain_min : Return the minimum RF transmission gain. get gain_max : Return the maximum RF transmission gain. If the return value is -123, it is -12.3 dB. HD Radio™ files are not supported. For HD Radio™ files, refer to the get power command. |

| Command | get rf_out_level <parameter> |
| --- | --- |
| Support | Weiver 1.0 : O Weiver Player 1.0 : O Weiver 2.0 : O Weiver Player 2.0 : O HD Radio™ : X |
| Description | Return RF output power. <parameter> Arbitrary time value in seconds. The time value must be less than the total time. If the return value is -123, it is -12.3 dBm. Refer to the getplay_time_total command for the total time of the file. HD Radio™ files are not supported. For HD Radio™ files, refer to the get power command. |

| Command | set play_time_current <parameter> |
| --- | --- |
| Support | Weiver 1.0 : O Weiver Player 1.0 : O Weiver 2.0 : O Weiver Player 2.0 : O HD Radio™ : O |
| Description | Command to set the start position of the file to be played in seconds. <parameter> Arbitrary time value in seconds. |

| Command | get play_time_total get play_time_current |
| --- | --- |
| Support | Weiver 1.0 : O Weiver Player 1.0 : O Weiver 2.0 : O Weiver Player 2.0 : O HD Radio™ : O |
| Description | get play_time_total : Return the size of the file currently being played in seconds. get play_time_current : Return the location of the file currently being played in seconds. |

| Command | set play_section <parameter1> <parameter2> |
| --- | --- |
| Support | Weiver 1.0 : O Weiver Player 1.0 : O Weiver 2.0 : O Weiver Player 2.0 : O HD Radio™ : X |
| Description | Command to set interval repetition. <parameter1> The value of arbitrary time in seconds to start repeating the interval. <parameter2> The value of arbitrary time in seconds to end the interval iteration. |

| Command | get play_section |
| --- | --- |
| Support | Weiver 1.0 : O Weiver Player 1.0 : O Weiver 2.0 : O Weiver Player 2.0 : O HD Radio™ : X |
| Description | Return interval repetition in seconds. |

| Command | set spec_inv off set spec_inv on |
| --- | --- |
| Support | Weiver 1.0 : O Weiver Player 1.0 : O Weiver 2.0 : O Weiver Player 2.0 : O HD Radio™ : O |
| Description | set spec_inv off : It is transmitted without being inverted left and right. set spec_inv on : It is transmitted in reverse left and right. |

| Command | get spec_inv |
| --- | --- |
| Support | Weiver 1.0 : O Weiver Player 1.0 : O Weiver 2.0 : O Weiver Player 2.0 : O HD Radio™ : O |
| Description | get spec_inv : Return the value of whether or not to be inverted. |

| Command | set sample_rate <parameter> set sample_rate <parameter> hz set sample_rate <parameter> khz set sample_rate <parameter> mhz |
| --- | --- |
| Support | Weiver 1.0 : O Weiver Player 1.0 : O Weiver 2.0 : O Weiver Player 2.0 : O HD Radio™ : X |
| Description | Command to adjust sample frequency <parameter> sample frequency The openfile command does not need to be changed because the sample frequency and center frequency of the file are automatically set. |

| Command | get sample_rate |
| --- | --- |
| Support | Weiver 1.0 : O Weiver Player 1.0 : O Weiver 2.0 : O Weiver Player 2.0 : O HD Radio™ : X |
| Description | Return the sample frequency |

| Command | set trigger_out_level low set trigger_out_level high get trigger_out_level |
| --- | --- |
| Support | Weiver 1.0 : X Weiver Player 1.0 : O Weiver 2.0 : O Weiver Player 2.0 : O HD Radio™ : O |
| Description | Command to control the output of the Trigger Out port. Set trigger_out_level : Low level (0V) is output. Set trigger_out_level high : High level (3.3V) is output. get trigger_out_level : Return the status value of the trigger out port. |

| Command | set extclk off set extclk on get extclk |
| --- | --- |
| Support | Weiver 1.0 : O Weiver Player 1.0 : O Weiver 2.0 : O Weiver Player 2.0 : O HD Radio™ : O |
| Description | command to set the 10 MHz synchronization of the equipment to the inside/outside. set extclkoff : Use the internal 10MHz of the equipment. set extclkon : Use an external input of 10 MHz. get extclk : Return the 10 MHz synchronization status value. |

| Command | set trigger_in off set trigger_in on get trigger_in |
| --- | --- |
| Support | Weiver 1.0 : O Weiver Player 1.0 : O Weiver 2.0 : O Weiver Player 2.0 : O HD Radio™ : O |
| Dscription | Command to set whether or not to operate multiple equipment according to the input level of the Trigger In Port. Set trigger_in off : It operates regardless of the input level of Trigger In Port. Set trigger_in on: It operates when the input level of Trigger In Port is High (3.3v). get trigger_in : Returns whether it operates according to motivation. |

| Command | set current_directory <parameter> |
| --- | --- |
| Support | Weiver 1.0 : O Weiver Player 1.0 : O Weiver 2.0 : O Weiver Player 2.0 : O HD Radio™ : O |
| Description | Command to specify the folder where the file of the equipment is located. <parameter> absolute path of the <parameter> folder. Please change the folder before using the openfile command. |

| Command | get current_directory |
| --- | --- |
| Support | Weiver 1.0 : O Weiver Player 1.0 : O Weiver 2.0 : O Weiver Player 2.0 : O HD Radio™ : O |
| Description | Return the current folder of the equipment. |

| Command | get file_count |
| --- | --- |
| Support | Weiver 1.0 : O Weiver Player 1.0 : O Weiver 2.0 : O Weiver Player 2.0 : O HD Radio™ : O |
| Description | Return the number of Weiver files and HD Radio™ files in the equipment's current folder. |

| Command | get filename <parameter> |
| --- | --- |
| Support | Weiver 1.0 : O Weiver Player 1.0 : O Weiver 2.0 : O Weiver Player 2.0 : O HD Radio™ : O |
| Description | Return the Weiver file and HD Radio™ file name of the index among the list of files retrieved from the current folder of the equipment. <parameter> Specify the index of the file list. The index must be less than the value returned from get file_count. |

| Command | get folder_count |
| --- | --- |
| Support | Weiver 1.0 : O Weiver Player 1.0 : O Weiver 2.0 : O Weiver Player 2.0 : O HD Radio™ : O |
| Description | Returns the number of sub folders from the equipment's current folder. |

| Command | get foldername <parameter> |
| --- | --- |
| Support | Weiver 1.0 : O Weiver Player 1.0 : O Weiver 2.0 : O Weiver Player 2.0 : O HD Radio™ : O |
| Description | Return the name of the sub folder of the equipment's current folder. <parameter> Specify the index of the folder. It must be less than the value returned from get folder_count. |

| Command | set power <parameter> |
| --- | --- |
| Support | Weiver 1.0 : X Weiver Player 1.0 : X Weiver 2.0 : X Weiver Player 2.0 : X HD Radio™ : O |
| Description | Command to adjust the RF transmission power of HD Radio™. <parameter> Arbitrary power value. When power -20.5 dBm is applied, <parameter> is -205. When gain -43.2 dBm is applied, <parameter> is -432. |

| Command | get power get power_min get power_max |
| --- | --- |
| Support | Weiver 1.0 : X Weiver Player 1.0 : X Weiver 2.0 : X Weiver Player 2.0 : X HD Radio™ : O |
| Description | Get power : Return the RF transmission power of HD Radio™. Get power_min : Return the minimum value of the RF transmission power of HD Radio™. Get power_max : Return the maximum value of the RF transmission power of HD Radio™. If the return value is -205, the transmission power is -20.5 dBm. If the return value is -423, the transmission power is -43.2 dBm. |

| Command | get freq_min get freq_max |
| --- | --- |
| Support | Weiver 1.0 : X Weiver Player 1.0 : X Weiver 2.0 : O Weiver Player 2.0 : O HD Radio™ : O |
| Description | get freq_min : return the minimum frequency value. get freq_max : return the maximum frequency value. |

[Example : When playing Weiver file]

connect 50000

set current_directory D:\WeiverData\Record\File

openfile lumantek-drivingtest-tdmb-agcon.wpj

play start

[Example : When playing HD Radio™ FM file]

connect 50000

set current_directory D:\WeiverData\Record\File

openfile CC_FMr440d_e1wfr013.bin

set freq 92900000

set power -300

play start

[Example : When playing HD Radio™ AM files]

connect 50000

set current_directory D:\WeiverData\Record\File

openfile IB_AMr208c_e1awfa11.bin

set freq 820000

set power -300

play start

[Command usage example]

connect, disconnect Command

How to use)

connect port number : Here, the port number refers to the port number that will receive a response after sending the packet.

disconnect : disconnect

Description)

When sending the connect command, you have to also inform the UDP port number to receive a response.

The UDP port number is specified as the same value as the u2_RecvPortFromWeiverPlayer value in the program example. The UDP port number is variable.

The receiving UDP port number of the equipment is fixed at 50504.

Sending disconnect stops the operation of the equipment.

Example)

1. connect 50000,

2. disconnect

rescandisk command

How to use)

When the rescandisk command is sent, the disk drive is updated in the equipment.

If you can't see the disk drive on the e-SATA, you can see an invisible disk drive when you refresh using the recandisk command.

Description)

Use if the disk drive is not renewed automatically.

Example)

1. rescandisk

set current_directory, get current_directory command

get file_count, get filename, get folder_count, get foldername command

openfile, get openfile command

How to use)

set current_directory directory : Here directory means the disk drive of the equipment.

get file_count : Gets the number of files in extension wpj.

get file name 'index' : Here the range of the index can have [number of files - 1].

get folder_count : get the number of folders in the set current_directory.

get folder name 'index' : Here the range of the index can have [number of folders - 1].

openfile 'file name': Here the file name means a file with extension wpj.

Description)

It specifies xxx.wpj you are trying to play.

If you are playing before using the openfile command, you have to stop.

Example)

1. set current_directory f:\

2. get current_directory

3. get file_count

4. get filename 0

5. get filename 1

6. get folder_count

7. get foldername 0

8. get foldername 1

9. openfile testrecordtest.wpj

10. get openfile

play start, play stop, play pause, play resume, get play_status command

How to use)

play start, play stop, play pause, play resume, get play_status

Description)

Select a file using the openfile command and use the play start, play stop, play pause, play resume command.

play start : Start Play

play stop : Stop Play

play pause : Pause Play

play resume : Resume paused Play

get play_status : Read current status

Example)

1. play start

2. get play_status

2. play pause

3. play resume

4. play stop

set freq, get freq command

How to use)

set freq frequency

Weiver 1.0 : Frequency is from 50000000(50MHz) to 1000000000(1GHz).

Weiver 2.0 : Regarding frequency, refer to ‘get freq_min’, ‘get freq_max’ command

Description)

After selecting a file using the openfile command, you can change the frequency using the set freq command. Units are Hz, KHz, and MHz.

Example)

1. set freq 400000000 (setting 400MHz)

2. get freq

3. set freq 400 MHz (setting 400MHz)

4. set freq 400000 KHz (setting 400MHz)

5. set freq 400000000 Hz (setting 400MHz)

get gain_min, get gain_max, get rf_out_level 0, get gain, set gain command

How to use)

Use when you want to give more Gain when playing after using openfile command.

Gain generally ranges from -30 dB to +30 dB. However, if you select the file to play, you can check the range of Gain for this file using the ‘get gain_min’, ‘get gain_max’ command.

getrf_out_level time : Here the range of time is the total play time of the file.

Description)

The unit is 0.1 dB.

For example, in order to give a gain of +10 dB, enter set gain 100.

How to use)

1. get gain_min

2. get gain_max

3. get rf_out_level 0

4. set gain 100

5. get rf_out_level 0

6. get gain

get play_time_total, get play_time_current, set play_time_current,

get play_section, set play_section command

How to use)

You can use the openfile command to select a file and move the location of the file you are going to play, or get time-related information.

The get play_time_total command displays the total time of the selected file in seconds.

The set play_time_current command continues to play around the loop when you specify the start and end times. The start time must be less than the end time and the end time must be less than the total time of the file.

Description)

The unit is seconds.

Here, the playable time of the play file stands for 630 seconds.

pp5 moves the file location to a 5-second point and plays it.

playrepeat 6 10 plays by looping between 6 and 10 seconds.

Example)

1. get play_time_total

2. get play_time_current

3. set play_time_current 5

4. set play_section 6 10

5. get play_section

set spec_inv off/on, get spec_inv command

How to use)

set spec_inv off : RF is transmitted as the spectrum of the file being played.

set spec_inv on : The spectrum of the file being played is inverted and RF is transmitted.

Description)

Use set spec_inv command in the case the spectrum needs to be reversed and transmitted.

Example)

1. set spec_inv off

2. set spec_inv on

set sample_rate, get sample_rate command

How to use)

set sample_rate sample frequency

Weiver 1.0 : sample frequency is from 5000000(5MHz) to 30000000(30MHz).

Weiver 2.0 : sample frequency is from 1250000(1.25MHz) to 70000000(70MHz).

The sample frequency stands for I/Q rate, and in the case of BW 8 MHz, I/Q rate is 10 MHz. In the case of BW 24 MHz, the I/Q rate is 30 MHz.

Description)

If you select a file using the openfile command, the sample frequency is automatically selected.

When manually modified, the rate can be varied using the set sample_rate command.

The unit is Hz.

Example)

1. set sample_rate 12000000, set sample_rate 12 mhz, set sample_rate 12000 khz

2. get sample_rate

set trigger_out_level, get trigger_out_level  command

How to use)

WeiverPlayer 1.0, Weiver 2.0, and WeiverPlayer 2.0 have Trigger Out ports.

set trigger_out_level low : A signal of LOW level (0V) is output to the trigger out port.

set trigger_out_level high : A signal of HIGH level (3.3V) is output to the trigger out port.

Description)

You can set the output level of the Trigger Out port.

Example)

1. set trigger_out_level low

2. set trigger_out_level high

set extclk, get extclk command

How to use)

You can synchronize the operation clock of the equipment with 10 MHz from the outside.

set extclk off : Use internal 10MHz.

set extclk on : Synchronize with 10 MHz input from outside.

Description)

Generally set extclk off is used.

Example)

1. set extclk off

2. set extclk on

3. get extclk

set trigger_in, get trigger_in command

How to use)

set trigger_in off : RF is transmitted regardless of the input level of trigger in Port.

Set trigger_in on : RF is transmitted when the input level of trigger in Port is HIGH (3.3V).

Description)

If you use ‘set trigger_in’ to play start, it is played according to the input level of trigger in Port. If you have multiple Weivers, you can synchronize and operate it.

Example)

1. set trigger_in off

2. set trigger_in on

3. get trigger_in

[WeiverPlayerRemoteAPI 소스]

CWinApp theApp;

using namespace std;

const unsigned short u2_WeiverPlayerPort = WEIVER_PLAYER_API_RX_PORT;

const unsigned short u2_RecvPortFromWeiverPlayer = 50000;

const char s1a_WeiverPlayerIP[] = "192.168.100.1";

//const char s1a_WeiverPlayerIP[] = "127.0.0.1";

//const char s1a_WeiverPlayerIP[] = "192.168.10.15";

int _tmain(int argc, TCHAR* argv[], TCHAR* envp[])

{

int nRetCode = 0;

if (!AfxWinInit(::GetModuleHandle(NULL), NULL, ::GetCommandLine(), 0))

{

_tprintf(_T("AfxWinInit failed.\n"));

nRetCode = 1;

}

else

{

wchar_t wCmdLine[1000];

wchar_t wSendCmd[1000];

WSADATA wsaData;

WSAStartup(MAKEWORD(2,2), &wsaData);

while(nRetCode == 0)

{

memset(wCmdLine, 0, 1000);

memset(wSendCmd, 0, 1000);

wprintf(_T("\nCMD>"));

_getws_s(wCmdLine, 1000);

if(wcsncmp(wCmdLine, _T("exit"), wcslen(_T("exit"))) == 0)

{

nRetCode = 1;

}

else

{

SendToStringCMD((char*)wCmdLine, 2*wcslen(wCmdLine));

}

wprintf(_T("\n"));

}

WSACleanup();

}

return nRetCode;

}

void SendToStringCMD(char *s1p_Data, unsigned int u4_Size)

{

SOCKET h_Socket;

SOCKADDR_IN Addr;

int SentBytes;

WvPlayerRemotePacket_t t_Packet;

WvPlayerRemotePacket_tp tp_Packet;

char *pSendData = (char *)&t_Packet;

char   message[2000];

memset(message, 0, 2000);

int clntAddrSize = sizeof(Addr);

h_Socket = socket(PF_INET, SOCK_DGRAM, IPPROTO_UDP);

memset(&Addr, 0, sizeof(Addr));

Addr.sin_family = AF_INET;

Addr.sin_port = htons(u2_RecvPortFromWeiverPlayer);

Addr.sin_addr.s_addr = htonl(INADDR_ANY);

struct timeval timeout;

timeout.tv_sec = 2;//2 seconds

timeout.tv_usec = 0;

int optlen = sizeof(timeout);

setsockopt(h_Socket, SOL_SOCKET, SO_RCVTIMEO, (const char *)&timeout, optlen);

if(bind(h_Socket, (SOCKADDR*)&Addr, sizeof(Addr)) == SOCKET_ERROR)

{

wprintf(_T("bind() Error"));

closesocket(h_Socket);

return;

}

t_Packet.u4_ID = WEIVER_PLAYER_ID;

t_Packet.u4_Reserved = WV_PLAYER_REMOTE_CMD_SET_STRING;

t_Packet.u4_DataSize = u4_Size;

memset(t_Packet.u1a_Data, 0, 1000);

memcpy(t_Packet.u1a_Data, s1p_Data, u4_Size);

memset(&Addr, 0, sizeof(Addr));

Addr.sin_family = AF_INET;

Addr.sin_port = htons(u2_WeiverPlayerPort);

Addr.sin_addr.s_addr = inet_addr(s1a_WeiverPlayerIP);

SentBytes = sendto(h_Socket

,pSendData

,4/*sizeof(t_Packet.u4_ID)*/

+ 4/*sizeof(t_Packet.u4_Reserved)*/

+ 4/*sizeof(t_Packet.u4_DataSize)*/ + u4_Size

,0, (SOCKADDR*)&Addr, sizeof(SOCKADDR_IN));

if(SentBytes == SOCKET_ERROR)

{

wprintf(_T("\nSend Fail"));

}

else

{

memset(&Addr, 0, sizeof(Addr));

int length = recvfrom(h_Socket

, message, 2000, 0, (SOCKADDR*)&Addr, &clntAddrSize);

if(length == -1)

{

wprintf(_T("Time Out"));

}

else

{

tp_Packet = (WvPlayerRemotePacket_tp)(message);

wprintf(_T("%s"), tp_Packet->u1a_Data);

}

}

closesocket(h_Socket);

}